## Station 1: Le Chatelier's Principle Challenge

Part A: Contact Process Analysis: You are a chemical engineer optimizing the Contact Process for sulfuric acid production.

2SO₂(g) + O₂(g) ⇌ 2SO₃(g)

Scenario 1: Your plant currently operates at 450°C and 2 atm pressure. The board of directors wants to increase profits by changing operating conditions.

- Pressure Analysis:

  - Predict what happens if pressure is increased to 10 atm

  - Explain your prediction using Le Chatelier's Principle

- Temperature Decision:

  - The reaction is exothermic (ΔH = -197 kJ mol⁻¹)

  - Predict the effect of decreasing temperature to 350°C on yield

  - Predict the effect on reaction rate

  - What temperature compromise would you recommend?

Part B: Methanol Synthesis Challenge: Your company is switching to methanol production.

CO(g) + 2H₂(g) ⇌ CH₃OH(g)

- Triple Threat Analysis:

  - If you simultaneously increase pressure, decrease temperature, and remove methanol as it forms, predict the effect on equilibrium position.

  - Rank these changes from most effective to least effective for increasing methanol yield.

  - Justify your ranking.

## Station 2: Equilibrium Calculations Laboratory

Part A: Contact Process Equilibrium: At 500°C, an equilibrium mixture in the Contact Process contains:

2SO₂(g) + O₂(g) ⇌ 2SO₃(g)

- [SO₂] = 0.300 mol dm⁻³

- [O₂] = 0.150 mol dm⁻³

- [SO₃] = 1.200 mol dm⁻³

- Calculate Kc for the Contact Process at 500°C

- Verify your answer by checking units (what should they be?)

Challenge Calculation: 3. A new reactor starts with [SO₂] = 0.500 mol dm⁻³, [O₂] = 0.250 mol dm⁻³, and [SO₃] = 0.100 mol dm⁻³

- Calculate the reaction quotient (Qc)

- Compare Qc to your calculated Kc

- Predict the direction of reaction

Part B: Methanol Synthesis Data Analysis: You have data from three different methanol production plants operating at different temperatures:

CO(g) + 2H₂(g) ⇌ CH₃OH(g)

| Temperature | Kc Value |
| --- | --- |
| 500 K | 14.5 mol⁻² dm⁶ |
| 550 K | 6.25 mol⁻² dm⁶ |
| 600 K | 2.94 mol⁻² dm⁶ |

- Plot Kc vs Temperature and describe the relationship

- Connect to thermodynamics: Explain why Kc decreases with increasing temperature for this reaction

- Industrial implications: Which temperature gives the best equilibrium yield? Why don't factories use this temperature?

## Station 3: Industrial Optimization Challenge

Part A: Contact Process Economics

You're consulting for a sulfuric acid plant. Current conditions:

2SO₂(g) + O₂(g) ⇌ 2SO₃(g)

SO₃(g) + H₂O(l) → H₂SO₄(aq)

- Temperature: 450°C (catalyst: V₂O₅)

- Pressure: 1-2 atm

- Conversion rate: 98% per pass

Optimization Scenarios:

- The Pressure Dilemma:

  - Le Chatelier's Principle suggests higher pressure increases yield

  - However, plants use only 1-2 atm. Research the industrial data provided and explain why

- Catalyst Investigation:

  - Why is vanadium pentoxide (V₂O₅) used instead of cheaper alternatives?

  - How does the catalyst affect the equilibrium vs. the reaction rate?

- Multi-stage Process:

  - The Contact Process uses multiple reaction stages. Using equilibrium principles, explain why this is more efficient than a single large reactor

Part B: Methanol Plant Design:

Design decisions for a new methanol plant:

CO(g) + 2H₂(g) ⇌ CH₃OH(g)

- Current industrial conditions: 250°C, 50-100 atm, copper-zinc catalyst

- Pressure Economics:

  - Why do methanol plants use such high pressure when the Contact Process uses low pressure?

  - Calculate the cost-benefit: High pressure equipment costs $2M extra but increases yield by 15%. Annual revenue increase = $800K. Is this worthwhile?

- Temperature Optimization:

  - Your data shows maximum yield at 200°C, but plants use 250°C

  - Use the rate vs. yield concept to explain this compromise

- Catalyst Considerations:

  - Research shows a new catalyst works at 150°C. What equilibrium advantages would this provide?

  - What practical challenges might arise?

## Station 4: Environmental and Social Impact Analysis

Part A: Sulfuric Acid's Global Role

Sulfuric acid is the world's most produced chemical (~200 million tons annually).

2SO₂(g) + O₂(g) ⇌ 2SO₃(g)

SO₃(g) + H₂O(l) → H₂SO₄(aq)

- Applications Research:

  - List 4 major uses of sulfuric acid.

  - Explain why sulfuric acid's properties make it suitable for each use listed.

- Environmental Challenge:

  - SO₂ emissions from Contact Process plants cause acid rain.

  - Modern plants achieve 99.5% conversion. Using Le Chatelier's Principle, explain how removing SO₂ from emissions affects equilibrium and its implication on acid rain production.

- Sustainability Analysis:

  - Research shows newer plants use "double absorption" - SO₃ is removed and converted to H₂SO₄, then remaining gases are processed again

  - Explain how this relates to continuous removal of products and equilibrium shifts

Part B: Methanol and the Future

Methanol is increasingly important as a fuel and chemical feedstock.

CO(g) + 2H₂(g) ⇌ CH₃OH(g)

- Carbon Cycle Connection:

  - Modern methanol synthesis uses CO from various sources

  - How could using CO₂ (from carbon capture) instead of CO affect the equilibrium? Consider: CO₂(g) + 3H₂(g) ⇌ CH₃OH(g) + H₂O(g)

- Green Hydrogen Impact:

  - "Green hydrogen" from renewable energy could replace fossil-fuel-derived H₂

  - How would this change the overall sustainability without affecting the equilibrium chemistry?

- Methanol Economy Concept:

  - Research the "methanol economy" concept proposed by George Olah

  - How do equilibrium principles support or challenge the feasibility of methanol as a primary fuel?

## Station 5: Real-World Problem Solving

Part A: Contact Process Troubleshooting

Crisis Scenario: You're the chief chemist at a sulfuric acid plant. Production has dropped by 30% over the past week.

2SO₂(g) + O₂(g) ⇌ 2SO₃(g)

SO₃(g) + H₂O(l) → H₂SO₄(aq)

Available Data:

- Reactor temperature: Normal (450°C)

- Reactor pressure: Normal (1.8 atm)

- Feed gas composition: SO₂ reduced by 40%

- Catalyst bed: 6 months since replacement (normal replacement: 4-5 years)

- Root Cause Analysis:

  - Which factor is most likely causing the production drop?

  - Use equilibrium principles to explain how reduced SO₂ concentration affects the reaction

- Solution Strategies:

  - Propose 3 immediate actions to restore production

  - For each action, explain the equilibrium principle involved

- Quality Control:

  - If you increase SO₂ feed concentration by 50% above normal, predict the effects on:

    - Equilibrium position

    - Product purity

    - Unreacted gas recycling needs

Part B: Methanol Plant Innovation

Innovation Challenge: Your company wants to build a "micro-methanol" plant for remote locations.

Constraints:

- Must be 1/10th the size of traditional plants

- Cannot use high pressure (max 10 atm vs. normal 50-100 atm)

- Must achieve 80% of normal efficiency

- Equilibrium Engineering:

  - How does reducing pressure from 50 atm to 10 atm affect methanol yield?

  - Calculate the pressure effect: If Kp at 50 atm gives 90% conversion, estimate conversion at 10 atm

- Compensation Strategies:

  - What changes in temperature, catalyst, or process design could compensate for lower pressure?

  - Design a multi-stage process using equilibrium principles